UART-Based Digital Monitoring and Control System

Project Description

This project implements a UART-based digital monitoring and control system using Verilog HDL.

The system receives serial data through UART, processes the received data, generates a control output, and transmits the received data through UART.

The design is developed and verified using Xilinx Vivado.

Objectives

- To design a UART-based digital monitoring and            control system using Verilog HDL.

- To receive serial data through UART.

- To generate a control output based on the received data.

- To verify the design using simulation.

Design Description

The system consists of a Baud-Rate Clock Divider, UART Receiver, Control Logic, and UART Transmitter. The received serial data is converted into 8-bit digital data and processed to generate the required control output.

Design Code

`timescale 1ns / 1ps

module uart_monitoring_control (

input  wire       clk,

input  wire       reset,

input  wire       rx,

output reg        tx,

output reg        control_out,

output reg [7:0]  received_data

);

// Baud-rate clock divider

reg [3:0] baud_counter;

reg       baud_tick;

// UART receiver

reg [3:0] rx_bit_count;

reg [7:0] rx_shift_reg;

reg       rx_busy;

// UART transmitter

reg [3:0] tx_bit_count;

reg [7:0] tx_shift_reg;

reg       tx_busy;

// Baud tick generation

always @(posedge clk or posedge reset) begin

if (reset) begin

baud_counter <= 0;

baud_tick    <= 0;

end

else begin

if (baud_counter == 4'd9) begin

baud_counter <= 0;

baud_tick    <= 1;

end

else begin

baud_counter <= baud_counter + 1;

baud_tick    <= 0;

end

end

end

// UART Receiver

always @(posedge clk or posedge reset) begin

if (reset) begin

rx_bit_count  <= 0;

rx_shift_reg  <= 0;

rx_busy       <= 0;

received_data <= 0;

end

else if (baud_tick) begin

if (!rx_busy && !rx) begin

rx_busy      <= 1;

rx_bit_count <= 0;

end

else if (rx_busy) begin

rx_shift_reg[rx_bit_count] <= rx;

if (rx_bit_count == 4'd7) begin

received_data <= rx_shift_reg;

rx_busy       <= 0;

rx_bit_count  <= 0;

end

else begin

rx_bit_count <= rx_bit_count + 1;

end

end

end

end

// Control logic

always @(posedge clk or posedge reset) begin

if (reset)

control_out <= 0;

else if (received_data == 8'h01)

control_out <= 1;

else if (received_data == 8'h00)

control_out <= 0;

end

// UART Transmitter

always @(posedge clk or posedge reset) begin

if (reset) begin

tx <= 1;

tx_bit_count <= 0;

tx_shift_reg <= 0;

tx_busy      <= 0;

end

else if (!tx_busy && received_data != 0) begin

tx_busy      <= 1;

tx_bit_count <= 0;

tx_shift_reg <= received_data;

tx           <= 0;

end

else if (tx_busy && baud_tick) begin

if (tx_bit_count < 8) begin

tx <= tx_shift_reg[tx_bit_count];

tx_bit_count <= tx_bit_count + 1;

end

else begin

tx <= 1;

tx_busy <= 0;

tx_bit_count <= 0;

end

end

end

endmodule

Testbench Code

`timescale 1ns / 1ps

module tb_uart_monitoring_control;

reg clk;

reg reset;

reg rx;

wire tx;

wire control_out;

wire [7:0] received_data;

uart_monitoring_control DUT (

.clk(clk),

.reset(reset),

.rx(rx),

.tx(tx),

.control_out(control_out),

.received_data(received_data)

);

// Clock: 10 ns period

always #5 clk = ~clk;

initial begin

clk = 0;

reset = 1;

rx = 1;

#20;

reset = 0;

// Send start bit

#10 rx = 0;

// Send 8-bit data: 00000001

#100 rx = 1;

#100 rx = 0;

#100 rx = 0;

#100 rx = 0;

#100 rx = 0;

#100 rx = 0;

#100 rx = 0;

#100 rx = 0;

// Stop bit

#100 rx = 1;

#200;

$display("UART Simulation Completed");

$display("Received Data = %h", received_data);

$display("Control Output = %b", control_out);

$finish;

end

endmodule

Simulation Output

01_Simulation_Waveform.png

RTL Schematic

02_RTL_Schematic.png

Console Output

03_Console_Output.png

Power Analysis

04_Power_Analysis.png

Conclusion

The UART-Based Digital Monitoring and Control System was successfully designed using Verilog HDL. The design was verified through simulation and analyzed using RTL schematic, synthesis, implementation, and power analysis in Xilinx Vivado.

This project provided practical experience in UART communication, Verilog HDL, digital design, and the VLSI design flow.